EE 316 Computer Engineering Junior Lab

Design Project 6

Spring 2025

Specification:  	Digital system using Xilinx ZynQ processor

Due Date:      		Thursday, April 24

Friday, April 26 - written report due.

Parts List:

- Digilent Cora-Z7-07S

- PMOD buttons and LEDs

- Arduino Starter Kit components:

  - Photoresistors(s)

  - Potentiometer

  - Ultrasonic sensor (HC-SC04)

  - Servo Motor (SG90)

  - DC motor

  - Passive buzzer

  - LCD module (I2C)

  - 3.3/5V Power Supply

Develop a digital system that can sample 8-bit data from any of the available analog inputs on Digilent’s Cora Z7 on the Analog header. The system will then control the angular position of a servo motor, the intensity of PMOD LEDs, and the speed of a DC motor. The design will meet the following specifications:

- Use Digilentinc's Cora-Z7 board, along with Xilinx's Vivado and SDK tools for design.

- Utilize Xilinx's Zynq processor and other IPs to design the system, assign pins, generate the bitstream, and export it to Xilinx SDK.

- A potentiometer will provide a voltage between 0 to 3.3V with a 3.3V power supply to control the servo and the PMOD LEDs.

- The servo will use a PWM signal with a period of 20ms and duty cycles ranging from 2.5% to 12.5%. The PWM signal will be generated using a custom PWM IP, which will control the duty cycle using XADC values. The servo will rotate from -90 degrees to +90 degrees, as controlled by the potentiometer.

- The PMOD LEDs' light intensity will be controlled using the AXI custom IP PWM output.

- A light-sensitive resistor will create a voltage divider with another resistor using the 3.3V power supply to provide another analog voltage source. This photo-resistor circuit will control the DC motor and a buzzer.

- The DC motor will be controlled using the AXI Timer's PWM output.

- The passive buzzer will be controlled by the AXI Timer's PWM output.

- The design will use onboard buttons using AXI_GPIO IP blocks. The button inputs must use interrupts.

  - Pressing button BTN0 will reset the system asynchronously.

  - Pressing button BTN1 will select between two of the analog sources.

  - Pressing button BTN2 will select between system-enable and disable.

- The AXI Timer should provide periodic interrupts for all timing needs, such as one-second delays.

- The I2C LCD unit will display the status (Enable/Disable/Reset, etc.) on the first line and the analog source on the second line. Use a custom IP for this unit.

- The ultrasonic sensor is used as a protection mechanism to warn the system user when an object moves close to the system. The system shall use Xilinx AXI_Timer’s capture mode to determine the measure the width of the ECHO pulse that is proportional to twice the distance from sensor to the object that reflects the burst of sound back. It shall alert users by turning on colored LED lights & playing a siren (alarm). If the object is within 1 foot from the sensor, the dc-motor, and the servo should stop.

  - Tri-color LED: The ultrasonic sensor shall alert the user with a green LED that a moving object is further away by 2 feet from sensor. If the object moves closer than 2 feet, LED shall turn yellow and it will turn red with it is at a distance of 1 foot. The intensity will increase as the distance becomes smaller.

  - Sound alarm: The system shall alert the user with a sound alarm that uses the passive buzzer when the object moves closer than 2 feet. The buzzer frequency shall vary from 1000 Hz to about 3000 Hz as the distance closes to 1 foot.

  - LCD Display: The distance shall be displayed on the first line of the LCD display.   You are free to choose other useful information on the second line.

- To debug the design, it is recommended to use a debug logic analyzer as a tool. To demonstrate the functionality, a simple data transaction should be carried out on a selected bus. However, it is important to note that it may not be possible to add all signals on the AXI4 bus, so only important signals should be selected to show data transactions.

Useful Links:

- MicrozedChronicles:https://www.adiuvoengineering.com/microzed-chronicles-archive

- Zynq TRM

- Zynq Architecture

- AXI XADC

- AXI Timer

- https://cdn.sparkfun.com/datasheets/Sensors/Proximity/HCSR04.pdf

- https://randomnerdtutorials.com/complete-guide-for-ultrasonic-sensor-hc-sr04/

Teams:

| Team 1 | Team 2 | Team 3 | Team 4 | Writer |
| --- | --- | --- | --- | --- |
| Adamson | Isaak | Elkins | Carroll |  |
|  | Uon | Nevin - D'Allaird | Scott |  |
| Campo | Zalepeski | Afolayanka | Lamontagne |  |